“Positioning RICA for Conservation Agriculture Leadership”

# TECHNICAL SPECIFICATIONS SHEET – M&I WORKSHOP EQUIPMENT

NOTE TO BIDDERS

- You must complete all the empty spaces in the technical specification sheet to confirm your specifications being offered.

- Your bid must include a manufacturer’s brochure or leaflet describing the equipment being offered to enable your specifications to be verified.

- All brochures and leaflets submitted must be written in the English language or a translation into the English language.

- LATHE MACHINE

| Item No. | Technical Specifications Required: LATHE MACHINE | Technical Specifications Offered |
| --- | --- | --- |
| 1.0 | General |  |
| 1.1 | Machine type: Conventional horizontal lathe |  |
| 1.2 | Heavy-duty cast iron bed with high rigidity and minimal vibration |  |
| 1.3 | Center distance: minimum 1000-1500 mm |  |
| 1.4 | Turning diameter over bed: minimum 350 - 400 mm |  |
| 1.5 | Turning diameter over support: minimum 200-300 mm |  |
| 1.6 | Bed width: minimum 200-300 mm |  |
| 1.7 | Spindle speed range: minimum 40-1800 rpm |  |
| 1.8 | Spindle bore: minimum 38 mm |  |
| 1.9 | Spindle mount: industrial standard (e.g., Camlock or equivalent) |  |
| 1.10 | Main motor power: minimum 2.0-3.0 kW |  |
| 1.11 | Machine weight: minimum 800-1400 kg |  |
| 1.12 | Provision for coolant system and chip management |  |
| 2.0 | Headstock and Feed |  |
| 2.1 | Hardened and ground gears for smooth and quiet operation |  |
| 2.2 | Feed X-axis: approx. 0.015-0.40 mm/rev |  |
| 2.3 | Feed Z-axis: approx. 0.04 - 1.0 mm/rev |  |
| 2.4 | Metric threading capability: approx. 0.4-10 mm |  |
| 2.5 | Imperial threading capability: approx. 4 -56 TPI |  |
| 3.0 | Tailstock |  |
| 3.1 | Tailstock quill diameter: minimum 45 mm |  |
| 3.2 | Tailstock taper: MT3 or equivalent |  |
| 3.3 | Quill stroke: minimum 120 mm |  |
| 3.4 | Provision for taper turning adjustment |  |
| 4.0 | Electrical Requirements |  |
| 4.1 | Voltage: 230V (single-phase) or 400V (three-phase) |  |
| 4.2 | Frequency: 50 Hz |  |
| 4.3 | Machine must be compatible with Rwanda electrical standards |  |
| 4.4 | Electrical components must include protection against overload and short circuit |  |
| 5.0 | Accessories |  |
| 5.1 | 3-jaw chuck and 4-jaw chuck |  |
| 5.2 | Face plate |  |
| 5.3 | Steady rest and follow rest |  |
| 5.4 | Coolant system |  |
| 5.5 | Toolbox with standard tools |  |
| 5.6 | Work light and safety guards |  |
| 5.7 | Chip tray and splash guard |  |
| 6.0 | Manuals |  |
| 6.1 | Operator and maintenance manuals in English (hard copy and/or soft copy) |  |
| 7.0 | Spare Parts |  |
| 7.1 | List of recommended spare parts for at least 2 years of operation to be provided |  |
| 8.0 | Warranty |  |
| 8.1 | Minimum 12 months or 2000 operating hours, whichever comes first |  |
| 9.0 | After-sales Service |  |
| 9.1 | Supplier must provide local service support or authorized service agent |  |
| 9.2 | Installation, commissioning and basic operator training to be included |  |

# MILLING AND DRILLING MACHINE

| Item No. | Technical Specifications Required: MILLING & DRILLING MACHINE | Technical Specifications Offered |
| --- | --- | --- |
| 1.0 | General |  |
| 1.1 | Universal milling and drilling machine with vertical and horizontal spindle |  |
| 1.2 | Table size: minimum 1000 x 240 mm – 1500x250 mm |  |
| 1.3 | Table load capacity: minimum 150 kg |  |
| 1.4 | X-axis travel: minimum 500 mm |  |
| 1.5 | Y-axis travel: minimum 150 mm |  |
| 1.6 | Z-axis travel: minimum 300 mm |  |
| 1.7 | Vertical spindle speed: approx. 100-2000 rpm |  |
| 1.8 | Horizontal spindle speed: approx. 60-1350 rpm |  |
| 1.9 | Spindle taper: ISO/SK40 or equivalent |  |
| 1.10 | Motor power: ≥1.5 kW (vertical) and ≥2.0 kW (horizontal) |  |
| 2.0 | Electrical Requirements |  |
| 2.1 | Voltage: 230V / 400V |  |
| 2.2 | Frequency: 50 Hz |  |
| 3.0 | Accessories |  |
| 3.1 | Drill chuck, arbors, coolant system |  |
| 3.2 | Work light and standard tool kit |  |
| 4.0 | Warranty |  |
| 4.1 | Minimum 12 months |  |

# VERTICAL BAND SAW

| Item No. | Technical Specifications Required: VERTICAL BAND SAW | Technical Specifications Offered |
| --- | --- | --- |
| 1.0 | General |  |
| 1.1 | Vertical band saw for metal cutting |  |
| 1.2 | Table size: minimum 500 x 400 mm |  |
| 1.3 | Cutting capacity: approx. 180 x 300 mm |  |
| 1.4 | Blade speed: variable 10-180 m/min |  |
| 1.5 | Blade width range: approx. 3-16 mm |  |
| 1.6 | Motor power: ≥0.5 kW |  |
| 1.7 | Rigid frame and adjustable blade guide |  |
| 2.0 | Electrical Requirements |  |
| 2.1 | Voltage: 230V / 400V |  |
| 2.2 | Frequency: 50 Hz |  |
| 3.0 | Accessories |  |
| 3.1 | Blade welder, coolant system |  |
| 3.2 | Work light and material stop |  |
| 4.0 | Warranty |  |
| 4.1 | Minimum 12 months |  |

# SHEET METAL FOLDING/ BENDING MACHINE

| Item No. | Technical Specifications Required: Sheet metal bending machine | Technical Specifications Offered |
| --- | --- | --- |
| 1.0 | General |  |
| 1.1 | Manual sheet metal folding machine |  |
| 1.2 | Working length: minimum 1000-1500 mm |  |
| 1.3 | Max sheet thickness: minimum 2.5 mm |  |
| 1.4 | Bending angle: up to 135° |  |
| 1.5 | Segmented upper tool for flexibility |  |
| 1.6 | Adjustable angle stop |  |
| 1.7 | Robust steel construction |  |
| 2.0 | Electrical Requirements |  |
| 2.1 | Not applicable or minimal (manual operation) |  |
| 3.0 | Accessories |  |
| 3.1 | Segmented tools set |  |
| 3.2 | Operator manual |  |
| 4.0 | Warranty |  |
| 4.1 | Minimum 12 months |  |